HDF5 Data Storage Guide

Microscopy FLIM Analysis Application

# 1. Overview

This document describes the HDF5-based storage architecture for the FLIM Cell Analyzer application. The design replaces the prior approach of saving individual .tif files per cell with a single consolidated file per experiment, eliminating data duplication and dramatically improving both disk usage and access speed.

The core principle is: store original data once, extract any cell on the fly using the segmentation mask. A single cell’s pixel data is simply image[labels == cell_id] — an array indexing operation that takes microseconds.

# 2. Why HDF5

HDF5 (Hierarchical Data Format version 5) is a binary file format designed for large, complex scientific data. It is the established standard across microscopy, bioimaging, and physics. The following table compares storage approaches:

| Approach | Disk Usage | Access Speed | Interoperability |
| --- | --- | --- | --- |
| Individual .tif per cell | Very high (redundant pixel data, file overhead per crop) | Slow (filesystem I/O per cell, thousands of files) | Universal but unwieldy |
| Single HDF5 per experiment | Low (one copy of image data, gzip compressed) | Fast (chunked reads, single file handle) | Fiji, MATLAB, Python, R, C++ |
| Zarr directory | Low (similar compression) | Fast (parallel chunk reads, cloud-friendly) | Python-centric, growing adoption |

HDF5 is recommended as the primary format for this application because it produces a single portable file, is natively readable by Fiji/ImageJ and MATLAB, supports chunked compression out of the box, and has mature Python bindings via the h5py library. Zarr is a strong alternative if you later need cloud storage or terabyte-scale datasets.

## 2.1 Key HDF5 Features

Chunked storage: Data is divided into fixed-size blocks (chunks) on disk. When you read a slice of the array, only the chunks that overlap with your slice are loaded from disk — not the entire array. This is what makes single-cell extraction fast even from a multi-gigabyte file.

Compression: Each chunk is independently compressed. The gzip filter typically achieves 5–10× compression on FLIM decay data (which is sparse — most time bins are near zero). LZF is faster but compresses less; gzip is the better default for archival storage.

Hierarchical groups: An HDF5 file has an internal directory structure (like a filesystem within a file). Datasets can be organized into groups and subgroups, and each can carry arbitrary metadata attributes.

Lazy loading: Opening an HDF5 file does not load any array data into memory. Datasets are accessed like numpy arrays but data is only read from disk when you index into them. This means opening a 10 GB experiment file is instantaneous.

# 3. File Structure

Each experiment produces a single .h5 file with the following internal layout:

experiment_001.h5

│

├── intensity              dataset   (H, W)       float32 or uint16

├── decay                  dataset   (H, W, T)    uint16

├── labels                 dataset   (H, W)       int32

│

├── phasor/                group

│   ├── g                  dataset   (H, W)       float32

│   └── s                  dataset   (H, W)       float32

│

├── measurements           dataset   CSV string (pandas DataFrame)

│

└── metadata/              group

├── source_file        attr      original filename

├── laser_frequency    attr      float (Hz)

├── time_resolution    attr      float (ps per bin)

├── cellpose_model     attr      string (e.g. 'cyto3')

└── cellpose_diameter  attr      float or null

## 3.1 Dataset Descriptions

| Dataset | Shape | Dtype | Description |
| --- | --- | --- | --- |
| intensity | (H, W) | float32 | Sum-projected intensity image. Used as the primary display layer in napari and as the intensity_image argument to regionprops. |
| decay | (H, W, T) | uint16 | Raw FLIM photon-count histograms. T is the number of time bins (typically 64–256). This is the largest dataset and benefits most from chunking and compression. |
| labels | (H, W) | int32 | Cellpose segmentation mask. Each pixel’s value is the cell ID (0 = background). Used for single-cell extraction and ROI overlay in napari. |
| phasor/g | (H, W) | float32 | Phasor G coordinate map (cosine component of the Fourier transform at the first harmonic). |
| phasor/s | (H, W) | float32 | Phasor S coordinate map (sine component). |
| measurements | N rows | string | Per-cell measurements stored as a CSV string. Columns include label, area, mean_intensity, centroid-0, centroid-1, eccentricity, solidity, g_mean, s_mean, and bbox coordinates. |

# 4. Chunking and Compression Strategy

Chunking determines how data is physically laid out on disk and directly affects read performance. The optimal chunk shape depends on how you access the data.

## 4.1 Recommended Chunk Sizes

| Dataset | Chunk Shape | Rationale | Compression |
| --- | --- | --- | --- |
| intensity | (256, 256) | Spatial tiles — good for bounding-box cell crops | gzip, level 4 |
| decay | (128, 128, T) | Keep time axis contiguous so reading a pixel’s full decay is one chunk read | gzip, level 4 |
| labels | (256, 256) | Matches intensity chunks for co-located reads | gzip, level 4 |
| phasor/g, s | (256, 256) | Spatial tiles | gzip, level 4 |

The general rule: make chunk boundaries align with your access pattern. Since most operations are spatial (extract a cell’s bounding box), square spatial chunks of 128–256 pixels work well. For the decay dataset, include the full time axis in each chunk so that reading a single pixel’s lifetime histogram does not require loading multiple chunks.

## 4.2 Compression Comparison

| Filter | Compression Ratio | Write Speed | Read Speed |
| --- | --- | --- | --- |
| None | 1× (baseline) | Fastest | Fastest |
| LZF | 2–4× typical | Very fast | Very fast |
| gzip (level 4) | 5–10× typical | Moderate | Fast |
| gzip (level 9) | 6–12× typical | Slow | Fast |

Recommendation: use gzip at level 4 as the default. It provides strong compression with acceptable write speed. Read speed is largely unaffected by compression level because decompression is fast. FLIM decay data compresses especially well because most time bins contain zeros or very small values.

# 5. Disk Space Estimates

The following estimates are for a typical 2048 × 2048 image with 256 time bins:

| Component | Uncompressed | gzip (est.) | % of Total |
| --- | --- | --- | --- |
| intensity (float32) | 16 MB | ~4 MB | 3% |
| decay (uint16, 256 bins) | 2,048 MB | ~150–300 MB | 90% |
| labels (int32) | 16 MB | ~1 MB | <1% |
| phasor g + s (float32) | 32 MB | ~10 MB | 5% |
| measurements (CSV) | <1 MB | <1 MB | <1% |
| Total per experiment | ~2,100 MB | ~170–320 MB | 100% |

Compare this with the individual .tif approach: 500 cells × ~100 KB per crop × multiple channels easily exceeds the compressed HDF5 size, and you still need to keep the original image. The HDF5 approach stores everything once with no duplication.

# 6. Python Implementation

The following class provides the complete read/write interface for the application. It wraps h5py with methods aligned to the application’s workflow.

## 6.1 ExperimentStore Class

import h5py

import numpy as np

import pandas as pd

from pathlib import Path

class ExperimentStore:

"""

Manages a single experiment's data in one HDF5 file.

Usage:

# Writing

store = ExperimentStore("exp_001.h5", mode="w")

store.save_image(intensity, decay)

store.save_labels(masks)

store.save_phasor(g, s)

store.save_measurements(df)

store.close()

# Reading

store = ExperimentStore("exp_001.h5", mode="r")

df = store.load_measurements()

crop, mask = store.get_cell_crop(cell_id=42)

store.close()

"""

def __init__(self, path: str, mode: str = "r"):

self.path = Path(path)

self.h5 = h5py.File(path, mode)

def save_image(self, intensity, decay=None, metadata=None):

"""Save intensity image and optional FLIM decay stack."""

self.h5.create_dataset(

"intensity", data=intensity,

compression="gzip", compression_opts=4,

chunks=(min(256, intensity.shape[0]),

min(256, intensity.shape[1])))

if decay is not None:

chunks = (min(128, decay.shape[0]),

min(128, decay.shape[1]),

decay.shape[2])

self.h5.create_dataset(

"decay", data=decay,

compression="gzip", compression_opts=4,

chunks=chunks)

if metadata:

grp = self.h5.require_group("metadata")

for key, val in metadata.items():

grp.attrs[key] = val

def save_labels(self, labels):

"""Save or overwrite segmentation mask."""

if "labels" in self.h5:

del self.h5["labels"]

self.h5.create_dataset(

"labels", data=labels,

compression="gzip", compression_opts=4,

chunks=(min(256, labels.shape[0]),

min(256, labels.shape[1])))

def save_phasor(self, g, s):

"""Save precomputed phasor coordinate maps."""

grp = self.h5.require_group("phasor")

for name, arr in [("g", g), ("s", s)]:

if name in grp:

del grp[name]

grp.create_dataset(

name, data=arr, compression="gzip")

def save_measurements(self, df: pd.DataFrame):

"""Store per-cell DataFrame as CSV string."""

if "measurements" in self.h5:

del self.h5["measurements"]

self.h5.create_dataset(

"measurements",

data=df.to_csv(index=False))

def load_measurements(self) -> pd.DataFrame:

"""Load per-cell measurements as DataFrame."""

from io import StringIO

csv_str = self.h5["measurements"][()].decode()

return pd.read_csv(StringIO(csv_str))

## 6.2 Single-Cell Extraction

The most performance-critical operation is extracting a single cell’s data. Two strategies are available, depending on whether bounding boxes have been precomputed.

def get_cell_crop(self, label_id, pad=5, bbox=None):

"""

Extract one cell's image data using the label mask.

Parameters

----------

label_id : int

Cell ID from the segmentation mask.

pad : int

Padding pixels around bounding box.

bbox : tuple, optional

(min_row, min_col, max_row, max_col) from

regionprops. If provided, skips the argwhere

scan for a ~100x speedup.

Returns

-------

crop : ndarray    Intensity crop (H_crop, W_crop)

mask : ndarray    Boolean mask within the crop

"""

labels = self.h5["labels"]

intensity = self.h5["intensity"]

if bbox is not None:

r_min, c_min, r_max, c_max = bbox

else:

coords = np.argwhere(labels[()] == label_id)

if len(coords) == 0:

return None, None

r_min, c_min = coords.min(axis=0)

r_max, c_max = coords.max(axis=0)

# Apply padding and clip to image bounds

r_min = max(0, r_min - pad)

c_min = max(0, c_min - pad)

r_max = min(labels.shape[0], r_max + pad)

c_max = min(labels.shape[1], c_max + pad)

# HDF5 reads only the chunks overlapping this slice

crop = intensity[r_min:r_max, c_min:c_max]

mask = labels[r_min:r_max, c_min:c_max] == label_id

return np.array(crop), np.array(mask)

Performance tip: always pass the bbox argument when you have it. The measurements DataFrame from regionprops_table includes bbox-0, bbox-1, bbox-2, bbox-3 columns. Using these avoids scanning the entire label mask with argwhere, reducing extraction time from milliseconds to microseconds.

## 6.3 FLIM Decay Extraction

For FLIM-specific analysis, you often need the raw decay histogram for a single cell:

def get_cell_decay(self, label_id, bbox=None, pad=5):

"""

Extract a cell's decay histograms.

Returns (decay_crop, mask) where decay_crop is

shape (H_crop, W_crop, T).

"""

if "decay" not in self.h5:

return None, None

# Reuse the bbox logic from get_cell_crop

crop, mask = self.get_cell_crop(

label_id, pad=pad, bbox=bbox)

if crop is None:

return None, None

# Read the corresponding decay data

r_min, c_min, r_max, c_max = ... # same bounds

decay_crop = np.array(self.h5["decay"]

[r_min:r_max, c_min:c_max, :])

return decay_crop, mask

# 7. Integration with the Application

The ExperimentStore connects to the application’s main window at two points: when opening/creating an experiment, and when analysis results are produced.

## 7.1 Opening an Experiment

# In MainWindow.open_image():

def open_image(self):

path, _ = QFileDialog.getOpenFileName(...)

# If it's an existing .h5 file, reopen previous work

if path.endswith(".h5"):

self.store = ExperimentStore(path, mode="r+")

intensity = self.store.h5["intensity"][()]

self.viewer.add_image(intensity, name="Intensity")

if "labels" in self.store.h5:

labels = self.store.h5["labels"][()]

self.viewer.add_labels(labels,

name="Cellpose Segmentation")

df = self.store.load_measurements()

self.data_model.set_measurements(df)

return

# Otherwise, load raw image and create new .h5

import tifffile

data = tifffile.imread(path)

h5_path = path.rsplit(".", 1)[0] + ".h5"

self.store = ExperimentStore(h5_path, mode="w")

...

## 7.2 After Segmentation

# In MainWindow._on_segmentation_done():

def _on_segmentation_done(self, masks):

self.store.save_labels(masks)

self.viewer.add_labels(masks, ...)

props = regionprops_table(masks, intensity,

properties=['label', 'area', 'mean_intensity',

'centroid', 'eccentricity',

'solidity', 'bbox'])

df = pd.DataFrame(props)

# ... add phasor metrics ...

self.store.save_measurements(df)

self.data_model.set_measurements(df)

With this pattern, closing and reopening the application picks up exactly where you left off. All segmentation results, measurements, and phasor data are persisted in the single .h5 file alongside the original image data.

# 8. Best Practices and Pitfalls

1. Always close the file. Use store.close() or a context manager (with block). An unclosed HDF5 file can become corrupted if the process crashes. Consider calling self.h5.flush() after each save operation for extra safety.

2. Do not open in write mode from multiple processes. HDF5 does not support concurrent writers. If you need multi-process access (unlikely for a desktop app), use mode='r' for readers and serialize all writes through a single process.

3. Store bounding boxes in the measurements table. Include the bbox property from regionprops. This eliminates the need to scan the full label mask when extracting individual cells, providing a ~100× speedup for random cell access.

4. Delete before overwriting. HDF5 does not reclaim space when you overwrite a dataset. Always del self.h5['dataset_name'] before creating a replacement. If file size becomes an issue after many overwrites, use h5py.File.copy() or the h5repack command-line tool to compact the file.

5. Use float32, not float64, for phasor and intensity data. Microscopy data rarely needs 64-bit precision. Using float32 halves memory usage and file size with no meaningful loss of accuracy.

6. Attach metadata as HDF5 attributes. Store experimental parameters (laser frequency, pixel dwell time, objective, date) as attributes on the metadata group. This keeps all provenance information in the same file as the data.

# 9. Dependencies

| Package | Install | Purpose |
| --- | --- | --- |
| h5py | pip install h5py | Python interface to HDF5. Core dependency for this storage layer. |
| numpy | pip install numpy | Array operations. Already a dependency of napari and cellpose. |
| pandas | pip install pandas | DataFrame for per-cell measurements. Serialized as CSV into HDF5. |
| hdf5plugin (optional) | pip install hdf5plugin | Additional compression filters (Blosc, Zstd, LZ4). Not required but useful for benchmarking. |